Jwdee Alamam S23108505

Sedra Aljundi S23108200

Kamar Bdair S23108164

# Project Summary Report

## High-level Description of the Organization

This project simulates a multi-site organization with three main branches: Site 1, Site 2, and Site 3. Each branch has its LAN containing PCs, servers, and switches. WAN links interconnect the branches through routers. At any one time, the organization expects about 50+ users stretched all over the sites. Essential services such as DHCP, DNS, and HTTP that allow dynamic addressing, name resolution, and web access will be provided inside the network. This network aims to provide seamless communication between sites, managing services and users efficiently at each location

Network Topology

The network follows a hierarchical topology combining WAN and LAN structures:

- WAN links (red lines) connect the three sites using routers (2811).

- LANs at each site use switches (2960 and 3560 models) to connect user PCs and servers.

This design ensures efficient communication inside each LAN and secure routing between sites.

## End Devices - PCs and Servers

PCs (PC-PT):

- They are meant to represent all organization users.

- Connected to local switches.

- Automatic IP address assignment uses DHCP.

- Access DNS and other HTTP services from servers.

Servers (Server-PT):

- At Site 1: Sets up one as a DHCP Server (DHCP-1, IP: 192.168.4.2).

- At Site 2: Sets a server as DNS Server (Bing.com DNS, IP: 192.168.5.2).

Sets one server as HTTP Server (Google.com HTTP, IP: 192.168.5.3).

- At Site 3: Sets one server as DHCP Server (DHCP-2, IP: 192.168.3.2).

Note: The DNS Server resolves domain names such as "bing.com" into IP addresses for PCs. The HTTP Server here serves as a host for a basic website to simulate internal/external websites.

## Intermediate Devices (Switches and Routers)

Switches:

- 2960-24TT and 2960-24:

Regular layer 2 switches connecting end devices at sites 1 and 3.

Distribute network traffic among multiple PCs.

- 3560-24PS:

A Layer 3 switch at Site 2.

Supports VLANs and routing if needed internally.

Connects servers and PCs more efficiently.

Routers (2811):

- Connect sites together.

- Handle routing between networks (different IP subnets).

- Each router interfaces with one LAN (local site) and two WAN links (other sites).

- Assigned specific WAN IP addresses to allow communication between sites

## IP Addressing Plan

- WAN Network between Routers:

site1 to site2:

192.168.1.1 (site1 router) ↔ 192.168.1.2 (site2 router)

site2 to site3:

192.168.2.1 (site2 router) ↔ 192.168.2.2 (site3 router)

- LANs:

Site 1 LAN: 192.168.4.0/24

Gateway: 192.168.4.1 (site1 router)

Site 2 LAN: 192.168.5.0/24

Gateway: 192.168.5.1 (site2 router)

Site 3 LAN: 192.168.3.0/24

Gateway: 192.168.3.1 (site3 router)

Each LAN has its own DHCP server or static IP assignments for essential servers and gateways.

## Services and features

- DHCP:

Automatically assigns the IP address to the PCs from Site 1 and Site 3.

It minimizes manual IP configuration efforts.

- DNS:

It resolves domain names into IP addresses.

It is necessary for user friendliness, as well for simulating external web access.

- HTTP:

It provides basic website hosting.

Simulates an internal web service or as an external Internet service.

- Routing:

Static routes or dynamic routing protocols can be used (depending on the configuration of the project).

It ensures communication between different LANs.

## How the Network Was Built (Steps)

- Routers configured with WAN IP addresses and LAN IP gateways.

- Switches added to each site and connected to PCs and servers.

- Servers configured with:

DHCP pools (for DHCP Servers).

DNS settings (for DNS Server).

Website hosting (for HTTP Server).

- Static or dynamic routing set between routers to allow inter-site communication.

- PCs configured to receive DHCP addresses.

- Testing done by:

Pinging between PCs across sites.

Accessing web server (HTTP).

Resolving domain names via DNS.

## Screenshot of Final Work:

## Screenshots Demonstrating Working of the Network: